L20 DOORS/SHUTTERS/HATCHES 
490 AUTOMATIC DOORS:

Manufacturer:

ASSA ABLOY Entrance Systems Limited, Unit 9 Windmill Business Village, Sunbury-on-Thames, Middlesex, TW16 7DY. Tel: 0333 006 3443. Email: architect.uk.entrance@assaabloy.com

Web: www.assaabloyentrance.co.uk

Project ref:      					Contact:

Models:

ASSA ABLOY SW300 – a heavy duty, fire rated, digitally controlled and intelligent swing door operator. Operator automatically opens a door inwards or outwards and can also be opened manually with a maximum of 22N opening force. Tested to 1m cycles. Verified with 3rd party EPD documentation. Operator including normal installation to comply with BS7036-0/EN16005.

Certification:
Manufacturer’s Declaration of Conformity

Environmental Product Declaration in accordance with ISO 14025 and EN 15804

Tested to: EN 60335-1:2012+A11:2014, EN 60335-2-103:2015, EN 61000-6-2:2005,                                                                EN 61000-6-3:2007+A1:2011, EN ISO 13849-1:2015, EN 16005:2012/AC:2015

Door Operator:

ASSA ABLOY SW300 swing door operator

- Doors to be opened: One single leaf OR Two double leaves with intelligent door coordinator

- Direction: Inward swing OR Outward swing OR Double action swing

- Operator Mounting: Transom mounted OR Door mounted OR Wall mounted

- Finish: Silver anodized OR Special anodized ref: XXXX OR ASSA ABLOY powder coat finish RAL ref: XXXX

- Recommended max. door weight and door width

- Inertia J = Door Weight x (door width)²/3

- Push Arm: 140 kgm² OR Pull Arm: 80 kgm²

Operation and control system:

- Suitable for staff trained and public environments.

- Stack pressure torque compensation with consistent door speeds.

- Wind force torque dampening with consistent door speeds.

- Automated latch retry for security doors.

- Intelligent power assist for manual operation below 20N.

- Activation: ASSA ABLOY uni-directional radar entry/exit activator OR ASSA ABLOY ESP elbow Push pads for entry/exit, with Power Assist Mode OR ASSA ABLOY Access Control for entry and ESP elbow push pad for exit OR Power assist mode – door is manually opened with assistance.

- Morning entry switch: Yes OR Not required

- Monitored battery backup (up to 300 activations): Required OR Not required

- Operation mode selector: Keyed ASSA ABLOY 4 mode; Open, Closed, 2-Way, 1-Way

- Electric locking required:  Fail safe OR Fail secure OR Not required

- Monitored door mounted safety devices: Standard OR Incorporating finger trap protection OR Not required

- Finger trap protection: Roller blind OR Incorporated within door mounted safety devices OR Door with finger guard built-in

- Safety barriers: One OR Two OR Not required

Dimensions:

- One single door leaf operator Length (Min 840mm): XXXXmm

- Two double door leaves operator Length (Min 1640mm): XXXXmm

- Height 70 mm

- Depth 148 mm

Warranty:

Standard 1 year warranty

Warranties can be extended subject to quotation from our service sales team. Contact: 0845 450 4633 service.uk.entrance@assaabloy.com

Please consult ASSA ABLOY Entrance Systems Limited technical literature for further details and the quotation for project specific notes.